GURU GOBIND SINGH INDRAPRASTHA UNIVERSITY B.Tech – Industrial Internet of Things (IIoT)

PROJECT REPORT

MQTT-Based Environmental Monitoring System Using ESP32-CAM, DHT22, MQ2, Mosquitto Broker & MQTT Explorer

Submitted By LAKSHYA B.Tech – Industrial IoT GGSIP University

ESP32-CAM ¦ DHT22 ¦ MQ2 ¦ Mosquitto MQTT Broker ¦ MQTT Explorer

# 1. Introduction

This project is a beginner-level Industrial IoT (IIoT) and Embedded Systems project designed to understand MQTT communication, sensor telemetry, and real-time IoT data monitoring.

The system uses an ESP32-CAM microcontroller to collect environmental data from two sensors:

- DHT22 – Temperature & Humidity Sensor

- MQ2 – Gas / Smoke Sensor

The collected data is transmitted over WiFi using the MQTT protocol to a Mosquitto MQTT Broker running on a Windows PC. MQTT Explorer is used to visualize live telemetry data and sensor graphs.

# 2. Objectives

The main objectives of this project are:

- Learn MQTT protocol fundamentals

- Understand Publish-Subscribe communication model

- Build real-time IoT telemetry pipeline

- Learn Mosquitto MQTT Broker setup

- Understand JSON payload transmission

- Visualize real-time sensor data using MQTT Explorer

- Learn ESP32 WiFi-based communication

# 3. Project Architecture

## 3.1 Data Flow Diagram

| Layer | Component | Role |
| --- | --- | --- |
| Sensor Layer | DHT22 + MQ2 | Data Collection |
| Edge Device | ESP32-CAM | WiFi Publisher (MQTT Client) |
| Transport | WiFi 802.11 b/g/n | Wireless Communication |
| Broker Layer | Mosquitto MQTT Broker (PC) | Message Routing |
| Visualization | MQTT Explorer | Subscribe & Display Live Data |

## 3.2 MQTT Architecture

| MQTT Component | Device / Software Used |
| --- | --- |
| Publisher | ESP32-CAM |
| Broker | Mosquitto MQTT Broker (Windows PC) |
| Subscriber | MQTT Explorer |

# 4. Components Required

| Component | Quantity |
| --- | --- |
| ESP32-CAM Module | 1 |
| DHT22 Temperature & Humidity Sensor | 1 |
| MQ2 Gas / Smoke Sensor Module | 1 |
| Arduino UNO (USB-Serial Converter) | 1 |
| Breadboard | 1 |
| Jumper Wires | Multiple |
| 10 kΩ Pull-up Resistor | 1 |
| USB Cable | 1 |
| Windows PC / Laptop | 1 |

# 5. Software Used

| Software | Purpose |
| --- | --- |
| Arduino IDE | Programming ESP32-CAM |
| Mosquitto MQTT Broker | MQTT Broker (Message Router) |
| MQTT Explorer | MQTT Visualization & Subscriber |
| Windows CMD | MQTT Testing & Verification |

# 6. Required Arduino Libraries

| Library | Author |
| --- | --- |
| PubSubClient | Nick O'Leary |
| ArduinoJson | Benoit Blanchon |
| DHT Sensor Library | Adafruit |
| Adafruit Unified Sensor | Adafruit |

# 7. Sensor Connections

## 7.1 DHT22 Connections

| DHT22 Pin | ESP32-CAM Pin |
| --- | --- |
| VCC | 3.3V |
| DATA | GPIO 14 |
| GND | GND |

Important: Add a 10 kOhm pull-up resistor between DATA pin and 3.3V for reliable DHT22 operation.

## 7.2 MQ2 Connections

| MQ2 Pin | ESP32-CAM Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| D0 (Digital Out) | GPIO 13 |

Note: Only digital output (D0) is used. Analog output (A0) is not used because GPIO34 is not exposed on most ESP32-CAM boards.

# 8. ESP32-CAM Upload Connections (Arduino UNO as USB-Serial)

The Arduino UNO is used as a USB-to-Serial converter to upload code to the ESP32-CAM, since the ESP32-CAM has no onboard USB-Serial chip.

| Arduino UNO Pin | ESP32-CAM Pin |
| --- | --- |
| RESET | GND (disables ATmega328) |
| 5V | 5V |
| GND | GND |
| TX | U0R (RX of ESP32-CAM) |
| RX | U0T (TX of ESP32-CAM) |
| GPIO0 | GND (Flash Mode – remove after upload) |

Flash Mode: Connect GPIO0 to GND before clicking Upload. Remove this connection after upload is complete, then press the RESET button.

# 9. Arduino IDE Setup

## Step 1 – Install Arduino IDE

Download and install Arduino IDE from: https://www.arduino.cc/en/software

## Step 2 – Add ESP32 Board URL

Open File > Preferences and paste the following URL in 'Additional Boards Manager URLs':

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

## Step 3 – Install ESP32 Board Package

Go to Tools > Board > Boards Manager, search for 'ESP32', and install:

- esp32 by Espressif Systems

# 10. MQTT Topics Used

| MQTT Topic | Purpose |
| --- | --- |
| iiot/lakshya/station01/telemetry | Sensor telemetry (temp, humidity, MQ2) |
| iiot/lakshya/station01/alert | Smoke / gas detection alert |
| iiot/lakshya/station01/command | Future remote control commands |

# 11. Mosquitto MQTT Broker Setup

## Step 1 – Install Mosquitto

Download Mosquitto from https://mosquitto.org/download/ and install the Windows 64-bit version.

## Step 2 – Configure Mosquitto

Open C:\Program Files\mosquitto\mosquitto.conf as Administrator (via Notepad). Add the following lines at the end of the file:

listener 1883

allow_anonymous true

## Step 3 – Restart Mosquitto

Open CMD as Administrator and run:

net stop mosquitto

net start mosquitto

## Step 4 – Verify Broker is Running

netstat -an | find "1883"

Expected output: 0.0.0.0:1883

# 12. Windows Firewall Configuration

Open Windows Defender Firewall with Advanced Security and create a new Inbound Rule with the following settings:

- Rule Type: Port

- Protocol: TCP

- Port Number: 1883

- Action: Allow the connection

- Profile: Private Network

# 13. Find PC IP Address

Open CMD and run:

ipconfig

Find the IPv4 Address (e.g., 192.168.1.5). This IP address must be entered as mqtt_server in the ESP32-CAM source code.

# 14. MQTT Explorer Setup

Download MQTT Explorer from: https://mqtt-explorer.com/

| Field | Value |
| --- | --- |
| Host | localhost |
| Port | 1883 |
| Protocol | mqtt:// |

# 15. Mosquitto Testing Commands

## Subscriber Command

cd "C:\Program Files\mosquitto"

mosquitto_sub -h localhost -t "iiot/#" -v

## Publisher Command

cd "C:\Program Files\mosquitto"

mosquitto_pub -h localhost -t "iiot/test" -m "hello"

Expected output in subscriber window: iiot/test hello

# 16. ESP32-CAM Source Code

#include <WiFi.h>

#include <PubSubClient.h>

#include <ArduinoJson.h>

#include <DHT.h>

const char* ssid     = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

const char* mqtt_server = "192.168.1.5";

const int   mqtt_port   = 1883;

const char* topicTelemetry = "iiot/lakshya/station01/telemetry";

const char* topicAlert     = "iiot/lakshya/station01/alert";

#define DHTPIN   14

#define DHTTYPE  DHT22

DHT dht(DHTPIN, DHTTYPE);

#define MQ2_DIGITAL 13

WiFiClient   espClient;

PubSubClient client(espClient);

unsigned long lastPublish     = 0;

const long    publishInterval = 10000;

void connectWiFi() {

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }

Serial.println("WiFi Connected!");

}

void mqttCallback(char* topic, byte* payload, unsigned int length) {

Serial.print("Message received: "); Serial.println(topic);

}

void connectMQTT() {

while (!client.connected()) {

String clientID = "ESP32CAM-" + String(random(0xffff), HEX);

if (client.connect(clientID.c_str())) {

client.subscribe("iiot/lakshya/station01/command");

} else { delay(3000); }

}

}

void publishSensorData() {

float temperature = dht.readTemperature();

float humidity    = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) { Serial.println("DHT22 Read Failed!"); return; }

int  mq2State     = digitalRead(MQ2_DIGITAL);

bool smokeDetected = (mq2State == LOW);

StaticJsonDocument<256> doc;

doc["device"]      = "station01";

doc["temperature"] = temperature;

doc["humidity"]    = humidity;

doc["mq2_digital"] = mq2State;

doc["smoke_alert"] = smokeDetected;

doc["wifi_rssi"]   = WiFi.RSSI();

char payload[256];

serializeJson(doc, payload);

client.publish(topicTelemetry, payload);

if (smokeDetected) {

StaticJsonDocument<128> alertDoc;

alertDoc["alert"] = "SMOKE_DETECTED";

char alertPayload[128];

serializeJson(alertDoc, alertPayload);

client.publish(topicAlert, alertPayload);

}

}

void setup() {

Serial.begin(115200);

dht.begin();

pinMode(MQ2_DIGITAL, INPUT);

delay(30000); // MQ2 warm-up

connectWiFi();

client.setServer(mqtt_server, mqtt_port);

client.setCallback(mqttCallback);

connectMQTT();

}

void loop() {

if (WiFi.status() != WL_CONNECTED)  connectWiFi();

if (!client.connected())             connectMQTT();

client.loop();

unsigned long now = millis();

if (now - lastPublish >= publishInterval) { lastPublish = now; publishSensorData(); }

}

# 17. Upload Procedure

- Connect GPIO0 to GND (Flash Mode).

- Click Upload in Arduino IDE.

- When 'Connecting.....' appears, press the ESP32 RESET button once.

- After upload completes, disconnect GPIO0 from GND.

- Press RESET again to run the program.

# 18. Serial Monitor Output

Set baud rate to 115200. Expected output:

ESP32-CAM MQTT Project Starting

MQ2 Warming Up...

Connecting to WiFi...

WiFi Connected!

Connecting to MQTT Broker...

Connected!

Telemetry Published:

{"device":"station01","temperature":33.2,"humidity":60,"mq2_digital":1,"smoke_alert":false,"wifi_rssi":-54}

# 19. Example MQTT JSON Payload

{

"device"      : "station01",

"temperature" : 33.2,

"humidity"    : 60,

"mq2_digital" : 1,

"smoke_alert" : false,

"wifi_rssi"   : -54

}

# 20. Working Principle

- DHT22 sensor reads ambient temperature and humidity.

- MQ2 sensor detects smoke/gas presence via digital output (D0).

- ESP32-CAM connects to the local WiFi network.

- Every 10 seconds, ESP32-CAM publishes a JSON payload to the telemetry MQTT topic.

- If smoke is detected, an additional alert message is published to the alert topic.

- Mosquitto Broker receives and routes all published messages.

- MQTT Explorer subscribes and displays live telemetry graphs in real-time.

- Auto-reconnect logic handles WiFi and MQTT disconnections automatically.

# 21. Features

- Real-time MQTT communication over WiFi

- JSON-formatted sensor telemetry payload

- Smoke / gas detection with dedicated alert topic

- Live sensor graphs in MQTT Explorer

- Auto WiFi reconnect and Auto MQTT reconnect

- Low-cost hardware – ESP32-CAM with common sensors

- Expandable topic structure for future remote commands

# 22. Applications

- Smart environmental monitoring systems

- IoT telemetry and data logging systems

- Smoke and gas detection systems

- Home automation and industrial monitoring

- Industrial IoT (IIoT) learning platform

- MQTT protocol learning and experimentation

# 23. Limitations

- MQ2 uses only digital output – no precise gas concentration measurement

- No cloud database – sensor data is not stored persistently

- No TLS/SSL security – MQTT communication is unencrypted

- Local network only – not accessible from the internet

- Educational prototype – not production-grade

# 24. Future Improvements

| Improvement | Description |
| --- | --- |
| Cloud MQTT Broker | Deploy on HiveMQ or AWS IoT for remote access |
| Node-RED Dashboard | Build visual control panels with drag-drop flows |
| OLED Display | Show live sensor values on a local display |
| Relay / Fan Automation | Control fan or alarm based on threshold alerts |
| Secure MQTT (TLS) | Encrypt broker communication with SSL certificates |
| Database Logging | Store telemetry in InfluxDB or SQLite |
| OTA Firmware Update | Update ESP32 code over WiFi remotely |
| Mobile App Integration | View sensor data on Android / iOS app |

# 25. Learning Outcomes

This project provided practical understanding of:

- MQTT protocol fundamentals and QoS levels

- Publish/Subscribe architecture and topic hierarchy design

- Embedded WiFi communication using ESP32

- JSON telemetry payload design and transmission

- DHT22 and MQ2 sensor interfacing with ESP32-CAM

- Mosquitto Broker installation and configuration on Windows

- Real-time IoT data visualization using MQTT Explorer

- Auto-reconnect strategies for robust IoT systems

# 26. Conclusion

This project successfully demonstrates a beginner-level IoT telemetry system using the MQTT protocol. The ESP32-CAM collected real-time environmental sensor data from DHT22 (temperature & humidity) and MQ2 (smoke/gas) sensors, and transmitted it wirelessly to a Mosquitto MQTT Broker over WiFi.

MQTT Explorer was used to subscribe to the telemetry topics and visualize live data and sensor graphs. The project provided strong practical understanding of MQTT communication, the Publish-Subscribe architecture, JSON telemetry transmission, sensor interfacing with ESP32, and real-time IoT data monitoring.

This system serves as a solid foundational project for learning Embedded Systems and Industrial IoT (IIoT) concepts, and can be extended with cloud integration, security, and automation features in future iterations.